Firewall Lab   - Blocking  Port 80  (default HTTP port)              TEC 236

Down load the Firewall Software from:

http://www.planetlinks.com/tec236

Install the software using the defaults (requires a re-start)

Start the Firewall Software

Start > Program Files > Tiny Personal Firewall > Personal Firewall Engine

Start the Firewall Status

Start > Program Files > Tiny Personal Firewall > Personal Firewall Status

Minimize

Start the Firewall Administration

Start > Program Files > Tiny Personal Firewall > Personal Firewall Administration

Check the Firewall Enabled box

Set the slider bar to the middle setting – Ask Me First

Click the Advanced button

Uncheck the Ask for action when no rule is found

Click the Add button (add a port 80 rule)

Description:  HTTP

Protocol:  TCP and UDP

Port Type:  Single

Port Number:  80

Application: Any

Address type:  Any address

Port type:   Single port

Port number:  80

Rule valid:   Always

Action:   Deny

Log when this rule match:  Check

Display alert box when this rule match:  Check

OK

Apply

OK

Apply

Retrive your machines IP

Start > Run > cmd > ipconfig

Enter into IE browser: http://yourmachineIP

You should receive an alert thet access was blocked (make sure you are reading the most recent alert)

The browser should display  -	    The page cannot be displayed

Change the HTTP Firewall rule to  Permit

Start > Program Files > Tiny Personal Firewall > Personal Firewall Administration

Advanced

Double click the HTTP rule

Set the Action to   Permit

OK

Apply

OK

Apply

Enter into IE browser: http://yourmachineIP

You should receive an alert thet access was Permited (make sure you are reading the most recent alert)

The browser should display  the page

Identify the connection in the Firewall status window

Start > Program Files > Tiny Personal Firewall > Personal Firewall Status